DucoDoor Wall

Manufactured by: DUCO 'Ventilation & Sun Control'

DucoDoor Wall is a louvre door with the same louvres and profiles as the louvre wall.

DucoDoor Wall and the wall thus form a perfect unit, as the door is fully integrated into the wall.

## Features:

- Louvre type: Choice among almost the entire DucoWall range, details on request

- Door type: Both single and double doors possible

- Direction of pivot: Outward or inward opening (reinforced only possible with outward opening doors)

- Version: Left or right

- Useful height (UH):

  - Maximum UH: 3000 mm

- Useful width (UW):

  - Maximum UW single door: 1500 mm

  - Maximum UW double door: 3000 mm

- Hinges: Duco Pivot system top and bottom (hinge plate thickness: 8 mm)

- Lock height (LH):

  - Standard LH: 1064 mm

  - Depending on louvre step, adaptable on customer demand.

- Frame profile: 50/50

- Mullion:

  - 40/21 for DucoWall Screening louvres

  - 40/21 for DucoWall Solid louvres

  - All others with Duco clips.

- Hand protection:

  - Lock plate on the lock side of the door.

  - Lock shield opposite the lock plate.

- Door furniture:

  - Standard:	T-handle (outside) + door handle (inside)

Cylinder lock (with 3 keys) 60mm Type 30/30

  - Optional:	Panic lock - Outdoor module + Knob

Panic lock - Outdoor module (without knob)

Cylinder lock 40mm Type 30/10

  - Other combinations available on request.

- Visual free area: Idem selected louvre wall

- Physical free area: Idem selected louvre wall

- Material:

  - Aluminium: Al Mg Si 0.5

  - Plastic: Polyamide PA 6.6 - glass-fibre reinforced

## Surface treatment:

- Anodised in natural as standard (15-20 µm)

- Enamelled polyester powder coating (60-80 µm)

- Specific RAL codes and/or textured paint on request

## Functional specifications:

### Flow rate:

  - K-factor intake: dependent on louvre

  - K-factor exhaust: dependent on louvre